OPgain

Mechanical Assembly Guide

Achikam Dadon, Naama Sharabi, Shlomo Marmor, Shahar Ganani, Yinon Sasson

# OPgain

## Mechanical Assembly Guide

Table of Contents

OPgain	1

Mechanical Assembly Guide	1

Full Assembly Requires	1

Module Mechanical Assembly:	2

Step 1:	2

Step 2:	3

Step 3:	3

Mechanical Box Assembly:	3

Step 1:	3

Step 2:	3

Step 3:	4

Step 4:	4

Step 5:	5

Full Assembly Requires:

- 1 X Chassis with Inserts

- 1 X Chassis Top Cover

- 2 X Mounting Ear

- 10 X Heat Spreader (1 per module)

- 10 X Top Cover (1 per module)

- 3 X BushBar

- 2 X Bushbar Support

### Module Mechanical Assembly:

Bottom

Top

Step 1: Apply grease THERMAL GREASE TC2 20G ND to pads, marked with red arrows, on mechanics (top and bottom).

Step 2:  Assemble Module as shown in the image above using 4 DIN965 M3X10 screws.

Step 3:  Screw Modules into Mechanical Box using 4 ISO7045 M3X6 (PHILLIPS PAN HEAD) screws. (If using Mechanical Box)

### Mechanical Box Assembly:

Step 1:  Connect Rack Mount Ears to Mechanical Box (if relevant) using 3 DIN965 M3X6 screw as seen in the image above.

Step 2:  Voltage Assembly requires 3 cable plugs per module with 4mm hole (overall 30 each for ten channels) and recommended purchase of different color for connectivity clarity.
In the diagram below red denotes +12V, black -12V and yellow is GND.

Step 3: Place Bushbar on the BushBar Supports levels to create uniform voltage across each row with suggested placement of +12V to top, GND in the middle, and -12V on the bottom.

Step 4:  For cable plugs assembly 46 X ISO7045 M4X8 (PHILLIPS PAN HEAD) are needed for full assembly as seen in the image above.

Step 5:  For assembly of the BushBar to Mechanical Box Chassis 4 ISO4032 M4 are needed as seen in the image above.